***This sample code is for GT-521FXX modules from ADH-Tech***

***Please read before use***

There are two types of demo code due to MCU resource has limited, so we separate one is Basic Function and another are GetTemplate and SetTemplate. For menu handle, we always use software serial for communication of fingerprint module.

For GetTemplate demo that you must be changed SoftwareSerial libraries(512bytes) and restart IDE due to UART receiver buffer is 64bytes by default. We will store EEPROM on Arduino after get template. (only one template, it SRAM is too small)

SoftwareSerial libraries is located:

arduino-1.8.5\hardware\arduino\avr\libraries\SoftwareSerial\src

Caution: Remember get back SoftwareSerial libraries after use finish of  GetTemplate demo code.

1. Embedded: Arduino Uno

1. Install Arduino IDE 1.8.5 version or later.

2. Open Project.ino by IDE

3. Execute upload function as programming code

4. Execute tools\serial monitor and make sure configuration as

“No line ending”/ “9600bps”.

5. Demo starting

***History***

20190114-initial version

Basic function folder－

1.Enrollment

2.Identify

3.Delete All

GetTemplate folder－

1.GetTemplate PS.change softwareSerial libraries(512bytes version)

SetTemplate folder－

2.SetTemplate